# USER GUIDE: Skyline Data Processing & QC Script

## 1. Overview

This R script is designed to process consolidated Skyline datasets. It automates the transposition of data, performs data cleaning, calculates Quality Control (QC) metrics (R-squared and CV), and normalizes peak areas using both internal standards and total polar metabolite content.

## 2. Prerequisites

- R/RStudio: Ensure you have R and RStudio installed.

- Required Packages: The script requires dplyr and purrr. If not installed, run: install.packages(c("dplyr", "purrr"))

- Input File: A single consolidated Skyline .csv file where:

  - Column 1: Metabolite Names.

  - Subsequent Columns: Sample/Injection names.

## 3. Step-by-Step Instructions

### Step 1: Initialization and File Import

- Run the script in RStudio (Shortcut: Cmd + Option + R on Mac or Ctrl + Alt + R on Windows).

- A file selection window will appear. Select your consolidated Skyline .csv file.

- The script will automatically:

  - Set the working directory to the folder where your file is located.

  - Create a subfolder named RscriptOutput for all results.

  - Transpose the data so that Samples are rows and Metabolites are columns.

### Step 2: Sample Removal (Optional)

- A data viewer (View) will open showing the transposed data.

- The console will ask: should I remove any rows from subsequent analysis? (y/n):

- If you need to exclude specific samples (e.g., blanks, washes, or failed injections):

  - Type y and press Enter.

  - Enter unique identifiers for the rows to remove, separated by spaces (e.g., blank wash01 conditioning).

  - Confirm the list of identified rows by typing y.

### Step 3: Define Quality Control (QC) Samples

- CV Calculation: The script will ask for the names of your 1x Pool injections.

  - Prompt: pool1x names:

  - Input: Enter the exact keywords for your pools (e.g., pool1xa pool1xb pool1xc). You need at least 3 replicates for a valid CV.

- Dilution Curve: Enter the dilution factors used in the experiment.

  - Prompt: What pool dilutions were included in the experiment?

  - Input: Enter numbers separated by spaces (e.g., 1 0.33 0.1).

- R-squared Calculation: Follow the prompts to enter keywords for the injections used for the standard curve (e.g., pool1xa pool033xa pool01xa).

### Step 4: Normalization and Filtering

- Internal Standards: The script automatically identifies internal standards (e.g., 13C, 15N, D3, D4, aminopterin) and calculates a normalization vector.

- QC Thresholds: At the final stage, you will be prompted to set your QC cut-offs:

  - Rsq Cut-off: Enter the minimum R-squared value (e.g., 0.95).

  - CV Cut-off: Enter the maximum CV value (e.g., 0.30).

## 4. Output Files Description

All outputs are saved in the RscriptOutput folder:

- MSdataframe.csv: The initial transposed and cleaned raw data.

- filteredstandards.csv: Peak areas of identified internal standards.

- normalization_vector_fromstandards.csv: The scaling factors derived from internal standards.

- sampledata_normalized_withRsq_and_CV.csv: Standard-normalized data including calculated Rsq and CV for every metabolite.

- RsqCVfiltered_normalized_sampledata.csv: Data after removing metabolites that failed the Rsq/CV thresholds.

- finaldata.csv: The Final Result. Data normalized to total polar metabolites with QC metrics attached.